# TP 9 - Firewall Rules using snort and Mininet

LSI 3 : 2025/2026

Realisée par : 								encadré par :

Achraf SAbir 								Pr.IKRAM  Benabdelouahab

# Topology

Objectives

Part 1: Preparing the Virtual Environment Part 2: Firewall and IDS Logs

Part 3: Terminate and Clear Mininet Process

# Background / Scenario

In a secure production network, network alerts are generated by various types of devices such as security appliances, firewalls, IPS devices, routers, switches, servers, and more. The problem is that not all alerts are created equally. For example, alerts generated by a server and alerts generated by a firewall will be different and vary in content and format.

In this lab, to get familiar with firewall rules and IDS signatures.

# Required Resources

- Security Workstation virtual machine Password : cyberops

- Internet connection

Note: In this lab, the Security Workstation VM is a container for holding the Mininet environment shown in the Topology. If a memory error is received in an attempt to run any command, quit out of the step, go to the VM settings, and increase the memory. The default is 1 GB; try 2GB.

# Instructions

## Part 1: Preparing the Virtual Environment

- Launch Oracle VirtualBox and change the Security Workstation for Bridged mode, if necessary. Select Machine > Settings > Network. Under Attached To, select Bridged Adapter (or if you are using WiFi with a proxy, you may need NAT adapter) and click OK.

- Launch the Security Workstation VM, open a terminal and configure its network by executing the

configure_as_dhcp.sh script.

Because the script requires super-user privileges, provide the password for the user analyst : cyberops

[analyst@secOps ~]$ sudo ./lab.support.files/scripts/configure_as_dhcp.sh

[sudo] password for analyst: cyberops [analyst@secOps ~]$

- Use the ifconfig command to verify Security Workstation VM now has an IP address on your local network. You can also test connectivity to a public webserver by pinging www.cisco.com. Use Ctrl+C to stop the pings.

[analyst@secOps ~]$ ifconfig

enp0s3: flags=4163<UP,BROADCAST,RUNNING,MULTICAST> mtu 1500

inet 10.0.2.15 netmask 255.255.255.0 broadcast 10.0.2.255 inet6 fe80::a00:27ff:fe55:4407 prefixlen 64 scopeid 0x20<link> ether 08:00:27:55:44:07 txqueuelen 1000 (Ethernet)

RX packets 30 bytes 7662 (7.6 KB)

RX errors 0 dropped 0 overruns 0 frame 0

TX packets 53 bytes 7344 (7.3 KB)

TX errors 0 dropped 0 overruns 0 carrier 0 collisions 0 lo: flags=73<UP,LOOPBACK,RUNNING> mtu 65536

inet 127.0.0.1 netmask 255.0.0.0

## Part 2: Firewall and IDS Logs

Firewalls and Intrusion Detection Systems (IDS) are often deployed to partially automate the traffic monitoring task. Both firewalls and IDSs match incoming traffic against administrative rules. Firewalls usually compare the packet header against a rule set while IDSs often use the packet payload for rule set comparison. Because firewalls and IDSs apply the pre-defined rules to different portions of the IP packet, IDS and firewall rules have different structures.

While there is a difference in rule structure, some similarities between the components of the rules remain. For example, both firewall and IDS rules contain matching components and action components. Actions are taken after a match is found.

- Matching component - specifies the packet elements of interest, such as: packet source; the packet destination; transport layer protocols and ports; and data included in the packet payload.

- Action component - specifies what should be done with that packet that matches a component, such as: accept and forward the packet; drop the packet; or send the packet to a secondary rule set for further inspection.

A common firewall design is to drop packets by default while manually specifying what traffic should be allowed. Known as dropping-by-default, this design has the advantage protecting the network from unknown protocols and attacks. As part of this design, it is common to log the events of dropped packets since these are packets that were not explicitly allowed and therefore, infringe on the organization’s policies. Such events should be recorded for future analysis.

### Step 1: Real-Time IDS Log Monitoring

- From the Security Workstation VM, run the script to start mininet.

[analyst@secOps ~]$ sudo

./lab.support.files/scripts/cyberops_extended_topo_no_fw.py

[sudo] password for analyst:

*** Adding controller

*** Add switches

*** Add hosts

*** Add links

*** Starting network

*** Configuring hosts

R1 R4 H1 H2 H3 H4 H5 H6 H7 H8 H9 H10 H11

*** Starting controllers

*** Starting switches

*** Add routes

*** Post configure switches and hosts

*** Starting CLI: mininet>

The mininet prompt should be displayed, indicating mininet is ready for commands.

- From the mininet prompt, open a shell on R1 using the command below:

mininet> xterm R1

mininet>

The R1 shell opens in a terminal window with orange text and black background. What user is logged into that shell? What is the indicator of this?

the user is root , the indicator of this is the prompt ends with the # character

- From R1’s shell, start the Linux-based IDS, Snort.

[root@secOps analyst]# ./lab.support.files/scripts/start_snort.sh

Running in IDS mode

--== Initializing Snort ==-- Initializing Output Plugins! Initializing Preprocessors! Initializing Plug-ins!

Parsing Rules file "/etc/snort/snort.conf"

<output omitted>

Note: You will not see a prompt as Snort is now running in this window. If for any reason, Snort stops running and the [root@secOps analysts]# prompt is displayed, rerun the script to launch Snort. Snort must be running to capture alerts later in the lab.

- From the Security Workstation VM mininet prompt, open shells for hosts H5 and H10.

mininet> xterm H5

mininet> xterm H10

mininet>

- H10 will simulate a server on the Internet that is hosting malware. On H10, run the mal_server_start.sh

script to start the server.

[root@secOps analyst]# ./lab.support.files/scripts/mal_server_start.sh

[root@secOps analyst]#

- On H10, use netstat with the -tunpa options to verify that the web server is running. When used as shown below, netstat lists all ports currently assigned to services:

[root@secOps analyst]# netstat -tunpa

Active Internet connections (servers and established)

Proto  Recv-Q Send-Q	Local Address

tcp	0	0	0.0.0.0:6666

Foreign ddress

0.0.0.0:*

State

LISTEN

PID/Program name

2095/nginx: master

[root@secOps analyst]#

As seen by the output above, the lightweight webserver nginx is running and listening to connections on port TCP 6666.

- In the R1 terminal window, an instance of Snort is running. To enter more commands on R1, open another R1 terminal by entering the xterm R1 again in the Security Workstation VM terminal window. You may also want to arrange the terminal windows so that you can see and interact with each device.

- In the new R1 terminal tab, run the tail command with the -f option to monitor the /var/log/snort/alert file in real-time. This file is where snort is configured to record alerts.

[root@sec0ps analyst]# tail -f /var/log/snort/alert

Because no alerts were yet recorded, the log should be empty. However, if you have run this lab before, old alert entries may be shown. In either case, you will not receive a prompt after typing this command. This window will display alerts as they happen.

- From H5, use the wget command to download a file named W32.Nimda.Amm.exe. Designed to download content via HTTP, wget is a great tool for downloading files from web servers directly from the command line.

[root@secOps analyst]# wget 209.165.202.133:6666/W32.Nimda.Amm.exe

--2017-04-28 17:00:04-- http://209.165.202.133:6666/W32.Nimda.Amm.exe

Connecting to 209.165.202.133:6666... connected. HTTP request sent, awaiting response... 200 OK Length: 345088 (337K) [application/octet-stream] Saving to: 'W32.Nimda.Amm.exe'

W32.Nimda.Amm.exe 337.00K --.-KB/s

100%[==========================================>]

in 0.02s

2017-04-28 17:00:04 (16.4 MB/s) - 'W32.Nimda.Amm.exe' saved [345088/345088]

[root@secOps analyst]#

What port is used when communicating with the malware web server? What is the indicator?

the port is 52950. the indicator is the port forwarding from it to 6666 (see the screenshot)

Was the file completely downloaded?

yes it’s indicated by 100%

Did the IDS generate any alerts related to the file download?

YES ,

- As the malicious file was transiting R1, the IDS, Snort, was able to inspect its payload. The payload matched at least one of the signatures configured in Snort and triggered an alert on the second R1 terminal window (the tab where tail -f is running). The alert entry is show below. Your timestamp will be different:

04/28-17:00:04.092153 [**] [1:1000003:0] Malicious Server Hit! [**] [Priority: 0]

{TCP} 209.165.200.235:34484 -> 209.165.202.133:6666

Based on the alert shown above, what was the source and destination IPv4 addresses used in the transaction?

Source IP: 209.165.200.235 (The address on the left side of the arrow ->)

Destination IP: 209.165.202.133 (The address on the right side of the arrow ->)

Based on the alert shown above, what was the source and destination ports used in the transaction?

Source Port: 34484

Destination Port: 6666

Based on the alert shown above, when did the download take place?

04/28-17:00:04 (April 28th at 5:00:04 PM)

Based on the alert shown above, what was the message recorded by the IDS signature?

Malicious Server Hit!

On H5, use the tcpdump command to capture the event and download the malware file again so you can capture the transaction. Issue the following command below start the packet capture:

[root@secOps analyst]# tcpdump –i H5-eth0 –w nimda.download.pcap &

[1] 5633

[root@secOps analyst]# tcpdump: listening on H5-eth0, link-type EN10MB (Ethernet), capture size 262144 bytes

The command above instructs tcpdump to capture packets on interface H5-eth0 and save the capture to a file named nimda.download.pcap.

The & symbol at the end tells the shell to execute tcpdump in the background. Without this symbol, tcpdump would make the terminal unusable while it was running. Notice the [1] 5633; it indicates one process was sent to background and its process ID (PID) is 5366. Your PID will most likely be different.

- Press ENTER a few times to regain control of the shell while tcpdump runs in background.

- Now that tcpdump is capturing packets, download the malware again. On H5, re-run the command or use the up arrow to recall it from the command history facility.

[root@secOps analyst]# wget 209.165.202.133:6666/W32.Nimda.Amm.exe

--2017-05-02 10:26:50-- http://209.165.202.133:6666/W32.Nimda.Amm.exe

Connecting to 209.165.202.133:6666... connected. HTTP request sent, awaiting response... 200 OK Length: 345088 (337K) [application/octet-stream] Saving to: 'W32.Nimda.Amm.exe'

W32.Nimda.Amm.exe	100%[===================>] 337.00K --.-KB/s

in 0.003s

2017-05-02 10:26:50 (105 MB/s) - 'W32.Nimda.Amm.exe' saved [345088/345088]

- Stop the capture by bringing tcpdump to foreground with the fg command. Because tcpdump was the only process sent to background, there is no need to specify the PID. Stop the tcpdump process with Ctrl+C. The tcpdump process stops and displays a summary of the capture. The number of packets may be different for your capture.

[root@secOps analyst]# fg

tcpdump -i h5-eth0 -w nimda.download.pcap

^C316 packets captured

316 packets received by filter

0 packets dropped by kernel

[root@secOps analyst]#

- On H5, Use the ls command to verify the pcap file was in fact saved to disk and has size greater than zero:

[root@secOps analyst]# ls -l

total 1400

drwxr-xr-x 2 analyst analyst	4096 Sep 26 2014 Desktop

drwx------ 3 analyst analyst	4096 Jul 14 11:28 Downloads

drwxr-xr-x 8 analyst analyst	4096 Jul 25 16:27 lab.support.files

-rw-r--r-- 1 root	root	371784 Aug 17 14:48 nimda.download.pcap drwxr-xr-x 2 analyst analyst

-rw-r--r-- 1 root	root

-rw-r--r-- 1 root	root

4096 Mar 3 15:56 second_drive

345088 Apr 14 15:17 W32.Nimda.Amm.exe

345088 Apr 14 15:17 W32.Nimda.Amm.exe.1

[root@secOps analyst]#

Note: Your directory list may have a different mix of files, but you should still see the

nimda.download.pcap file.

How can be this PCAP file be useful to the security analyst?

is useful because it’s provide us Forensic Evidence,Deep Packet Inspection,Stream Reconstruction,Signature Development

### Step 2: Tuning Firewall Rules Based on IDS Alerts

In Step 1, you started an internet-based malicious server. To keep other users from reaching that server, it is recommended to block it in the edge firewall.

In this lab’s topology, R1 is not only running an IDS but also a very popular Linux-based firewall called iptables. In this step, you will block traffic to the malicious server identified in Step 1 by editing the firewall rules currently present in R1.

Note: While a comprehensive study of iptables is beyond the scope of this course, iptables basic logic and rule structure is fairly straight-forward.

The firewall iptables uses the concepts of chains and rules to filter traffic.

Traffic entering the firewall and destined to the firewall device itself is handled by the INPUT chain. Examples of this traffic are ping packets coming from any other device on any networks and sent to any one of the firewall’s interfaces.

Traffic originated in the firewall device itself and destined to somewhere else, is handled by the OUTPUT

chain. Examples of this traffic are ping responses generated by the firewall device itself.

Traffic originated somewhere else and passing through the firewall device is handled by the FORWARD

chain. Examples of this traffic are packets being routed by the firewall.

Each chain can have its own set of independent rules specifying how traffic is to be filtered for that chain. A chain can have practically any number of rules, including no rule at all.

Rules are created to check specific characteristics of packets, allowing administrators to create very comprehensive filters. If a packet doesn’t match a rule, the firewall moves on to the next rule and checks again. If a match is found, the firewall takes the action defined in the matching rule. If all rules in a chain have been checked and yet no match was found, the firewall takes the action specified in the chain’s policy, usually allow the packet to flow through or deny it.

- In the Security Workstation VM, start a third R1 terminal window.

mininet > xterm R1

- In the new R1 terminal window, use the iptables command to list the chains and their rules currently in use:

[root@secOps ~]# iptables -L -v

Chain INPUT (policy ACCEPT 0 packets, 0 bytes) pkts bytes target	prot opt

in	out	source	destination

Chain FORWARD (policy ACCEPT 6 packets, 504 bytes) pkts bytes target	prot opt

in	out	source	destination

Chain OUTPUT (policy ACCEPT 0 packets, 0 bytes) pkts bytes target	prot opt

in	out	source	destination

[root@secOps ~]#

What chains are currently in use by R1?

INPUT , FORWARD , OUTPUT

- Connections to the malicious server generate packets that must transverse the iptables firewall on R1. Packets traversing the firewall are handled by the FORWARD rule and therefore, that is the chain that will receive the blocking rule. To keep user computers from connecting to the malicious server identified in Step 1, add the following rule to the FORWARD chain on R1:

[root@secOps ~]# iptables -I FORWARD -p tcp -d 209.165.202.133 --dport 6666 - j DROP

[root@secOps ~]#

Where:

  - -I FORWARD: inserts a new rule in the FORWARD chain.

  - -p tcp: specifies the TCP protocol.

  - -d 209.165.202.133: specifies the packet’s destination

  - --dport 6666: specifies the destination port

  - -j DROP: set the action to drop.

- Use the iptables command again to ensure the rule was added to the FORWARD chain. The Security Workstation VM may take a few seconds to generate the output:

Chain OUTPUT (policy ACCEPT 0 packets, 0 bytes) pkts bytes target	prot opt

in	out	source	destination

[root@secOps analyst]#

tcp dpt:6666

209.165.202.133

anywhere

any

- any

tcp -

0 DROP

0

[root@secOps analyst]# iptables -L -v

Chain INPUT (policy ACCEPT 0 packets, 0 bytes)

pkts bytes target	prot opt

in	out	source	destination

Chain FORWARD (policy ACCEPT 0 packets, 0 bytes) pkts bytes target	prot opt

in	out	source	destination

- On H5, try to download the file again:

[root@secOps analyst]# wget 209.165.202.133:6666/W32.Nimda.Amm.exe

--2017-05-01 14:42:37-- http://209.165.202.133:6666/W32.Nimda.Amm.exe

Connecting to 209.165.202.133:6666... failed: Connection timed out. Retrying.

--2017-05-01 14:44:47-- (try: 2) http://209.165.202.133:6666/W32.Nimda.Amm.exe

Connecting to 209.165.202.133:6666... failed: Connection timed out. Retrying.

Enter Ctrl+C to cancel the download, if necessary.

Was the download successful this time? Explain.

No, the download was not successful. because the firewall rule (-j DROP) silently discarded the packets sent by H5 to the malicious server.

What would be a more aggressive but also valid approach when blocking the offending server?

A more aggressive approach would be to block all traffic to the IP address 209.165.202.133 entirely, rather than just blocking port 6666.Type your answers here.

## Part 3: Terminate and Clear Mininet Process

- Navigate to the terminal used to start Mininet. Terminate the Mininet by entering quit in the main Security Workstation VM terminal window.

- After quitting Mininet, clean up the processes started by Mininet. Enter the password cyberops when prompted.

[analyst@secOps scripts]$ sudo mn –c

[sudo] password for analyst:

CHECKLIST

Q1. Snort est-il en cours d’exécution ? En déduire son mode de fonctionnement (IDS ou IPS).

Commande à exécuter sur R1 :

ps aux | grep snort

Q2. Quelle interface réseau Snort surveille-t-il et pourquoi est-ce logique dans cette topologie ?

Commande à exécuter sur R1 :

snort -V

ou

cat /etc/snort/snort.conf | grep interface

Q3. Combien d’alertes ont été générées après le téléchargement du malware ?

Commande à exécuter sur R1 :

wc -l /var/log/snort/alert

Q4. Quel champ de la sortie permet de confirmer que la règle DROP est réellement utilisée ? Expliquez.

Commande à exécuter sur R1 :

iptables -L FORWARD -v -n

Q5. Test de connectivité ICMP après blocage : Le ping est-il bloqué ? Pourquoi ?

Commande à exécuter sur H5 :

ping 209.165.202.133

Q6. Analyse du fichier PCAP généré : Quel protocole de couche transport est observé dans la capture ?

Commande à exécuter sur H5 :

tcpdump -r nimda.download.pcap | head

## Q7. Identification de la direction du trafic dans l’alerte Snort

Basé sur la sortie : 209.165.200.235:34484 -> 209.165.202.133:6666

Quel hôte est le client et lequel est le serveur ? Justifier.

C’est 209.165.202.133:6666 car il est liée avec la port 6666

Q8. Quelle est la politique par défaut appliquée aux chains INPUT, OUTPUT et FORWARD ?

Commande à exécuter sur R1 :

iptables -S

Q9. Effet d’un arrêt de Snort sur la sécurité

Action : arrêter Snort (Ctrl+C dans le terminal Snort)

Le téléchargement du fichier malveillant est-il à nouveau possible ? Pourquoi ?

Non, le téléchargement n'est toujours pas possible. Pourquoi ? Snort agit comme un IDS passif qui surveille et alerte, mais ne bloque pas directement. Le blocage effectif a été configuré précédemment via iptables (le pare-feu). Même si vous arrêtez Snort, la règle iptables qui rejette les paquets (DROP) est toujours active dans le noyau Linux du routeur R1. Vous avez coupé "l'alarme" (Snort), mais la "porte blindée" (iptables) est toujours fermée.

## Q10. Blocage complet du serveur malveillant (tous les ports)

Objectif : Bloquer tout trafic TCP à destination du serveur malveillant, quel que soit le port.

-	Quel est le résultat du test et que peut-on conclure ?

Le téléchargement échoue toujours (timeout), car le pare-feu bloque les paquets. Conclusion : En supprimant la spécificité du port (par exemple en bloquant tout le trafic TCP venant/allant vers cette IP), la règle devient plus robuste. Si l'attaquant change le port de son serveur de malware (par exemple, passant du port 6666 au port 80 ou 443), le pare-feu continuera de le bloquer. C'est une approche "Défense en profondeur".

## Q12. Journalisation du trafic malveillant avant blocage

Objectif : Ajouter une règle qui journalise les tentatives d’accès au serveur malveillant avant de les

bloquer.

- Donnez la configuration réaliser pour la mise en place de ces règles sur R1

- Tester un téléchargement malveillant depuis H5, ensuite afficher les logs sur R1

- Quelle information apparaît dans les logs et pourquoi est-ce utile pour l’analyste sécurité ?

Pour que cela fonctionne, la règle de journalisation (LOG) doit être placée avant la règle de blocage (DROP) dans la liste, car iptables traite les règles séquentiellement. Configuration sur R1 : Il faut insérer la règle LOG en première position (index 1) : iptables -I FORWARD 1 -p tcp -d 209.165.202.133 -j LOG --log-prefix "Tentative Malware: " Affichage des logs sur R1 : Après avoir tenté le téléchargement depuis H5, lancez cette commande sur R1 pour voir les logs système : Bash tail -f /var/log/messages # OU selon la distribution : dmesg ¦ tail Information et utilité pour l'analyste : Les logs affichent : l'heure, l'IP source (H5), l'IP destination (Serveur Malware), le protocole et les ports. Utilité : Cela permet de distinguer un problème technique (réseau en panne) d'une action de sécurité volontaire (blocage pare-feu). Cela fournit aussi une preuve d'audit que des machines internes tentent de communiquer avec des entités malveillantes connues

## Q13. Quel est le rôle principal de Mininet dans ce TP ?

- Simuler des attaques réelles sur Internet

- Créer une topologie réseau virtuelle pour les tests

- Capturer le trafic réseau

- Configurer automatiquement les règles iptables

- Toutes les réponses sont correctes.

## Q14. À quoi peut servir le fichier nimda.download.pcap pour un analyste sécurité ?

- Restaurer le fichier supprimé

- Analyser le trafic réseau et confirmer l’attaque

- Mettre à jour les règles iptables automatiquement

- Redémarrer Snort

- Inutile

## Q15. Pourquoi Snort a-t-il généré une alerte lors du téléchargement du fichier ?

- Le port utilisé est interdit

- L’adresse IP source est inconnue

- Le payload correspond à une signature malveillante

- Le firewall a bloqué le paquet